October 9, 2008

Re: Automatic PLC Download

Download Instructions:

- Connect an RS-232 serial cable between the 9-pin serial connection on your computer and the 9-pin serial connector on the sheet feeder.

- Open the fploader_freev230 download software by double clicking on the “fploader_freev230” file.

Click “Browse” and select the new program file “sf_3_3_09”.

Click Next

Click Next

Click Next

Click Next, or if necessary set the correct COM port.  All other settings are correct.

Un-Check “clears the data within the PLC before download.   The other two boxes should be checked as shown above.  Then click Next

Click Execute

The program will automatically download.

This last screen shows that the file has been successfully downloaded.

Click End