Testing communication on the sheet feeder with asci xfers

- Restart you computer and turn on the sheet feeder.

- Start the Serial Hyper Terminal_fp0 in the asci xfers folder.

- Go to the File drop down and select properties.

- Make sure the com port is the correct one for your feeder.  Then click Configure.

- Check the settings so that they mach the following picture.

- Click Apply.

- Then click ok to exit the 2 dialog boxes.

- Click the Transfer drop down menu and select Send Text File.

- Direct the Look in box to the asci xfers folder and select the icut pick in 60 sec.txt file

10) once this is sent you should receive a display on the hyper terminal screen and the touch screen on the sheet feeder should be counting down from 60.